Educating our clients since 1989

# Limited moisture assessment report

Property: Summers Residence

Client: Deborah Summers

Report issued: March 30, 2026

Date(s) of assessment: March 11, 2026

Prepared by: Investigative Inspection Services, Inc. (IIS)

Consultant: Stanley Yeskolski

Table of Contents

Summary	3

General information	4

Introduction	5

Inspection equipment and standards	5

Instruments and equipment	7

Findings and discussion	10

Exterior	10

Interior	13

Attic	16

Crawl space	19

Basement	22

HVAC systems	25

Sampling results	28

Lab Results Summary	28

Direct Analysis of Tape-Lift Samples	28

Analysis of Air Samples	29

Analysis of dust for mycotoxins	30

Conclusion	36

Health-sensitive occupancy notice	38

Limitations	41

Attachments/appendices	43

# Summary

# General information

| Client Information: |  |  |  |
| --- | --- | --- | --- |
| Client name: | Deborah Summers | Email: | dboothsummers@gmail.com |
| Cell phone: | (804) 399-2014 | Work phone: | N/A |
| Client address: | 7306 Woodway Lane, Norfolk, VA 23505 |  |  |
| Inspection address: | 7306 Woodway Lane, Norfolk, VA 23505 |  |  |

| Assessment Details: |  |  |  |
| --- | --- | --- | --- |
| Inspector(s) present: | Stanley Yeskolski |  |  |
| Date(s) of assessment: | March 11, 2026 |  |  |
| Time(s) of assessment: | N/A |  |  |

| Environment: | Outside | First Floor | 2nd Floor | Basement |
| --- | --- | --- | --- | --- |
| CO2 measurement | N/A | N/A | N/A | N/A |
| Temperature: | N/A | N/A | N/A | N/A |
| Humidity: | N/A | N/A | N/A | N/A |
| Rainfall: | N/A |  |  |  |
| Conditions: | N/A |  |  |  |
| Particle counts | N/A |  |  |  |

| Building Details: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Construction date: |  |  | Approx. 1951 (approximately 75 years old) |  | Remodel date: | Rear addition present; date not reported |
| Number of elevations: |  |  | 4 |  | Number of stories: | 2 |
| Square footage: |  |  | N/A |  | Building location: | Norfolk, Virginia |
| Type of building: |  |  | Single-family residence |  |  |  |
| HVAC system |  |  | 2 Daikin systems and 1 Mitsubishi mini split; equipment located in the garage, attic, and wall-mounted locations |  |  |  |
| Landscape: |  | Grading generally slopes away from the home; drip and spray irrigation are present |  |  |  |  |
| Foundation: |  | Encapsulated crawl space with slab-supported rear addition |  |  |  |  |
| Construction exterior materials: |  |  |  | Primarily brick exterior with wood at the rear addition |  |  |
| Roof: | Mixed roof assembly; client reported prior repair to a flat roof area |  |  |  |  |  |
| Construction interior materials: |  |  |  | Hardwood flooring, plaster ceilings, wood paneling, and drywall finishes were observed or reported |  |  |
| Attic: | Conditioned attic with a dehumidifier, insulated rafter bays, and no soffit or ridge ventilation |  |  |  |  |  |

# Introduction

Investigative Inspection Services (IIS) was contacted to perform a limited moisture and mold assessment at 7306 Woodway Lane, Norfolk, Virginia 23505. The client reported chronic humidity, moisture, HVAC-related concerns, visible fungal growth, bubbling paint, ceiling cracking, and health symptoms that reportedly improve when the family is away from the home. A site visit occurred on March 11, 2026.

The scope of inspection included the following:

- A visual assessment of the exterior, first-story living room area, attic, crawl space, and accessible HVAC components to identify moisture issues, fungal reservoirs, structural concerns related to moisture exposure, and areas appropriate for targeted tape-lift sampling.

- Assessment of accessible moisture conditions and representative fungal contamination in areas of concern.

This report contains the following:

- An overview of the methodology and equipment used in this inspection, see Inspection Equipment and Standards

- Report of findings from the assessment, including laboratory results of tape-lift samples collected during the site visit.

- My expert opinion is based on the site visit observations, laboratory reports, and experience.

- Laboratory report of tape-lift samples for direct examination for fungi.

# Inspection equipment and standards

Visual inspection of the house included limited observation of the building and observations of building materials for the presence of suspected visible fungal growth or conditions conducive to fungal growth, as prescribed in the ASTM (Standard 2418-06) Baseline Survey Process. The survey did not include any destructive techniques and was limited to readily accessible areas/materials in the areas of concern. Destructive testing techniques were not used; therefore, hidden areas (for example: under/behind furniture, below insulation, behind wall coverings, under floor covering, and inside wall/ceiling cavities) could not be included in our scope of work.

# Instruments and equipment

| Instrument/Equipment Employed During the Assessment | Description | Utilized |
| --- | --- | --- |
| MOISTURE METERS |  |  |
| Tramex Survey Encounter | Non-invasive impedance moisture meter, up to ½” – ¾” depth |  |
| Tramex Roof/Wall Scanner | Non-invasive impedance moisture meter, up to 4” depth |  |
| FLIR MR59 Ball Probe Moisture Meter | Non-invasive capacitance moisture meter, up to 4” depth | Yes |
| Protimeter MMS2 | Combination pin-type/non-invasive moisture meter/hygrometer/thermometer |  |
| Delmhorst BD-10 Pin-type Moisture Meter | Pin-type resistance meter with probes | Yes |
| INDOOR AIR QUALITY |  |  |
| Extech RH300 Hygrometer/Thermometer | Measures environmental conditions - temperature and relative humidity |  |
| Supco IAQ55 CO₂/Humidity/Temperature Meter | Measures the quantity of CO₂ in the ambient air, as well as temperature and humidity |  |
| Particles Plus 8306 Handheld Particle Counter | Measures the quantity of various sizes of airborne particulates | Yes |
| Smoke Pen | Cotton wick inside of a pen-style holder, used to quickly determine the direction of airflow, if any |  |
| Shortridge Instruments ADM-870 Electronic Micromanometer | Measures air flow, velocity, pressure (and whether it is positive or negative), and temperature |  |
| IMAGING |  |  |
| Teslong NTS500 Industrial Endoscope | A borescope-style camera with a narrow lens is used for inspecting the inside of areas typically inaccessible for visual inspection |  |
| FLIR Thermal Camera Attachment for Android | Thermal imaging device used to detect invisible heat energy |  |
| BIOLOGICAL SAMPLING |  |  |
| Mycometer allergens, fungal levels | Collects airborne allergens and fungi, which are then analyzed for severity. |  |
| Tape Lift | A tape collects mold samples, then is sent to an accredited laboratory for evaluation | Yes |
| Air sampling |  |  |

The temperatures and relative humidity readings were obtained at various locations in subject rooms using the Particles Plus 8306 Handheld Particle Counter environmental sensor. Readings are obtained when starting the assessment and when entering the building after allowing sufficient time to acclimate. All temperature readings are in degrees Fahrenheit (˚F). All humidity readings are Relative Humidity (RH).

The relative moisture content (MC) of building materials potentially affected by the water was measured using a hand-held, non-invasive FLIR MR59 Ball Probe Moisture Meter. Moisture content is calculated by the meter as a relative reading comparing known dry materials to other materials. The non-invasive device is placed against the exterior or interior surface of the walls or floors and passed over the area being assessed while remaining in contact with the surface. The meter is designed to detect general areas of high moisture content within the substrate. They will not detect areas of decay and damage in the absence of moisture.

The probe moisture meter was the Delmhorst BD-10 with long probes. The moisture probe meter is used to probe substrates and framing for indications of soft or rotted materials and to obtain moisture content measurements. These meters use handheld twin probes that have insulated electrodes. The probes are electrically conductive at the tip for measuring the resistance of the material into which the tips are embedded. The meter displays moisture as a percentage by weight for wood and a relative percentage for gypsum. A moisture content of less than fifteen percent (15%) is considered to be “normal” MC, and less than the water activity required by fungi for growth.

Airborne particulates were measured using a Particles Plus Handheld 8306. This instrument utilizes a laser to measure up to six different channels (diameters of particulate measured in microns) of simultaneous data. A sample reading is taken outside to establish a baseline for the locality and then used in problem areas to assess particulate content. This instrument does not identify the particulate, only the quantity.

For the surface sampling, a tape lift was utilized.

# Findings and discussion

## Exterior

Rusting steel lintels were observed above multiple windows, with associated mortar separation and localized brick displacement. The garage lintel exhibited the greatest movement, with an approximate 1/4-inch separation at the left end. The right chimney flue had visible cracking, both chimneys had minimal mortar crown protection, and vegetation was growing near the caps. Corrosion staining at the copper flashing suggests incompatible ferrous fasteners may have been used. Crawl-space vents at the front left and left side are below grade, the front stoop trim was beginning to soften adjacent to the brick, and the wood-to-brick transition at the rear addition was not sealed, creating a potential air and moisture entry path.

A qualified masonry or exterior repair contractor should evaluate and repair the corroded lintels, repoint or replace separated mortar joints, repair chimney crown and flue defects, remove vegetation, and correct any incompatible flashing fasteners. Below-grade vent exposure, the unsealed addition joint, and the soft front stoop trim should be addressed to reduce continued moisture entry and air infiltration.

## Interior

At the transition between the first-story living room and the dining room addition, the living room floor was approximately 4 to 5 mm lower than the adjoining slab-supported area. No active staining was observed at the flooring or walls in this location during the assessment; however, the separation where the slab and pier-and-beam systems meet creates a pathway for crawl-space air to enter the occupied space. Prior fungal growth had reportedly been observed on furniture in this area. A tape-lift sample taken at the first-story floor returned light Aspergillus/Penicillium with trace mycelium, which is consistent with settled fungal contamination or low-level growth influence near the floor separation.

The floor separation should be evaluated by a qualified contractor and sealed after the underlying moisture and air-infiltration issues are addressed. Until permanent repairs are made, temporary sealing measures should be considered to reduce crawl-space air migration into the first-story living area. Affected furnishings and horizontal surfaces in this area should be cleaned or evaluated by a qualified mold remediation contractor following remediation of the crawl space and attic reservoirs.

## Attic

The attic had been converted to a conditioned space with a dehumidifier and did not have soffit or ridge ventilation. Wooden siding had been installed over the rafters and the rafter bays were insulated. Moisture readings taken through drilled access points did not show elevated moisture at the time of the assessment, and the openings were resealed with brown caulking. Even so, visible fungal growth was observed sporadically to heavily on the wood surfaces facing the attic air. Tape-lift analysis confirmed very heavy Penicillium with many mycelial fragments at the attic HVAC location and at the left attic ceiling sample, while the middle attic ceiling sample showed no fungi detected. The attic access also showed evidence of air leakage.

The attic requires professional mold remediation, including cleaning or removal of contaminated materials as determined by the remediation contractor. The attic access should be improved with a gasket or other air-sealing method, and humidity control should be maintained after remediation. Items stored in the attic should not be moved into occupied areas until they have been evaluated and, if appropriate, cleaned as potentially contaminated contents.

## Crawl space

The crawl space was difficult to access because of ductwork, plumbing, HVAC lines, and abandoned hydronic piping. Relative humidity ranged from 59% to 66% at approximately 74 degrees Fahrenheit. Visible fungal growth was observed throughout the crawl space, with coverage varying from light to heavy. Laboratory results confirmed very heavy Cladosporium and heavy to very heavy Aspergillus/Penicillium on crawl-space joists, with mycelial estimates ranging from few to many. A condensate line at the rear center was leaking at a taped tubing-to-PVC connection, leaving a small puddle below. The main trunk supply ducts were not insulated, exposed fiberglass was visible at a flex-duct connection, and the vapor barrier attachment was incomplete or inadequately fastened in several areas. At the front left, the sill plate and floor-joist bearing ends were severely deteriorated from apparent historic water intrusion at the vents. Although the wood appeared dry at the time of the assessment, the damage is structural in nature. A cut floor joist was also noted under the guest bathroom.

The crawl space requires professional mold remediation and corrective moisture control. The condensate leak should be repaired promptly, the crawl-space air barrier should be improved to reduce air migration into the home, and the vapor barrier should be properly secured where accessible. A qualified contractor should repair the deteriorated sill plate and joist bearing areas, evaluate the cut joist condition, and address below-grade vent exposure. Insulating the metal trunk ducts and improving dehumidifier air circulation, potentially through ducting or redistribution, would also be advisable.

## HVAC systems

Evidence of fungal contamination was observed within the HVAC system. A tape-lift sample from inside the air handler identified rare Sporidesmium, Alternaria, and Curvularia, while a separate insulation sample from the HVAC system showed no fungi detected. By contrast, the attic-side HVAC sample returned very heavy Penicillium with many mycelial fragments, confirming that fungal growth is present on HVAC-adjacent surfaces in the attic. These findings indicate that at least one HVAC system serving the affected areas has been impacted by fungal contamination even though not every sampled component showed growth.

The HVAC system serving the affected areas should be evaluated and restored by a qualified contractor familiar with mold-contaminated systems and the current NADCA standard. Contaminated components should be cleaned or replaced as appropriate after source removal has been completed in the attic and crawl space. The system should not be relied upon to restore indoor air quality until remediation and post-remediation cleaning are complete.

# Sampling results

## Lab Results Summary

Project: Summers Residence
Lab: Hayes Microbial Consulting, LLC
Report Number: 26011751
Collection Date: March 11, 2026
Analysis Date: March 12, 2026

Samples Collected: Nine tape-lift samples were collected from the HVAC system, the first-story floor area, the attic wood surfaces and attic-side HVAC location, and crawl-space framing.

Critical Findings: Very heavy Penicillium was identified at the attic HVAC and attic ceiling left sample locations. Crawl-space joists returned very heavy Cladosporium and heavy to very heavy Aspergillus/Penicillium. The first-story floor sample returned light Aspergillus/Penicillium, while the HVAC insulation sample and attic ceiling middle sample both showed no fungi detected.

## Direct Analysis of Tape-Lift Samples

Tape-lift samples were collected from selected surfaces exhibiting visible suspect microbial growth or areas of concern identified during the onsite assessment. A clear adhesive medium (Bio-Tape) was gently pressed onto the surface to capture spores, hyphae, and fungal structures, then submitted to an accredited microbiological laboratory for direct microscopic examination. This method is intended to confirm the presence of mold growth on the sampled surface, identify the predominant fungal types present, and estimate the relative level of surface contamination. Results are semi-quantitative and are interpreted in conjunction with visual observations, moisture measurements, and building history.

Laboratory Results and Interpretation

The laboratory reports spore loading using a standardized scale (Rare, Light, Moderate, Heavy, Very Heavy) and a mycelial estimate (None Detected, Trace, Few, Many) to help distinguish between simple spore deposition and active fungal colonization on the sampled surface. In this assessment, the following surfaces showed fungal growth consistent with colonization rather than normal background ecology:

Tape-Lift Sample Results (Hayes Microbial Report 26011751)

Sample 111, collected from inside the air handler, identified rare Sporidesmium, Alternaria, and Curvularia. Sample 222, collected from HVAC insulation, showed no fungi detected. Sample 333, collected at the first-story floor, identified light Aspergillus/Penicillium with trace mycelium.

Sample B4373308 from the attic-side HVAC location identified very heavy Penicillium with many mycelial fragments. Sample B4326704 from the attic ceiling middle showed no fungi detected. Sample B4296879 from the left side of the attic ceiling identified very heavy Penicillium with many mycelial fragments.

In the crawl space, sample B4326729 from a near-center joist identified very heavy Cladosporium with few mycelial fragments. Sample B4326696 from the center joist identified very heavy Aspergillus/Penicillium with many mycelial fragments, and sample B4343139 from the front center joist identified heavy Aspergillus/Penicillium with many mycelial fragments.

The attic and crawl-space samples are consistent with active fungal growth rather than incidental spore deposition. The combination of heavy to very heavy spore loading and the presence of mycelial fragments demonstrates that these areas contain established fungal reservoirs associated with chronic moisture exposure or prolonged humidity.

The light Aspergillus/Penicillium finding at the first-story floor supports the field observation that air and particulate migration from below the floor assembly is likely affecting the occupied space. Taken together with the attic and crawl-space results, the laboratory findings support the need for professional remediation, source control, and detailed cleaning of the affected living areas and HVAC components.

# Conclusion

A qualified licensed contractor should perform all repairs on the home. Any mold requiring professional remediation should be performed by a qualified licensed mold remediation contractor.

The inspection identified fungal growth and moisture-related defects in the attic, crawl space, HVAC system, and first-story living area. The most significant findings were heavy fungal reservoirs in the crawl space and attic, active or recent HVAC-related contamination, and a floor separation at the first story that can allow contaminated crawl-space air to enter the living space.

The crawl space contained widespread visible fungal growth and laboratory-confirmed heavy contamination on framing members, along with a leaking condensate connection, inadequate vapor-barrier attachment in areas, and structural deterioration at the front-left sill plate and joist bearing location. The attic also contained laboratory-confirmed heavy Penicillium growth, including at the attic-side HVAC location.

The first-story living room showed floor separation where the slab-supported addition meets the pier-and-beam framing, and this condition, combined with prior fungal growth on furnishings and the light Aspergillus/Penicillium floor sample, supports the conclusion that fungal particulates are migrating into occupied areas.

Immediate Action Required: Retain a qualified mold remediation contractor to remediate the attic, crawl space, contaminated HVAC components, and affected first-story contents or surfaces; repair the leaking condensate line; and implement temporary measures to reduce crawl-space air entry through the floor separation until permanent repairs are completed.

Secondary Priority: Retain qualified contractors to repair the structurally deteriorated crawl-space framing, improve crawl-space air sealing and vapor-barrier attachment, evaluate the cut joist and ongoing floor movement, and address the exterior masonry, chimney, and flashing defects that may contribute to future moisture problems.

The homeowner is strongly encouraged to address these items promptly to limit continued fungal amplification, reduce indoor exposure, and protect the structural components of the home.

Based on the visible conditions and laboratory findings, Condition 3 exists in the attic, crawl space, and portions of the HVAC system. Condition 2 is also likely present in the first-story living area and on contents that have been exposed to migrated fungal particulates. After remediation, the home and affected HVAC components should be thoroughly cleaned and verified dry before rebuilding or returning stored contents to use.

The following standards and guidelines establish the minimum requirements for this project:

- ANSI/IICRC S520:2024 - Standard for Professional Mold Remediation (4th Edition)

- ANSI/IICRC S500:2021 - Standard for Professional Water Damage Restoration (5th Edition)

- ACR 2025 - The NADCA Standard for Assessment, Cleaning and Restoration of HVAC Systems

- EPA Guidelines - Mold Cleanup After Disasters and Indoor Environmental Guidelines

- OSHA Standards - Worker respiratory protection and occupant safety protocols (29 CFR 1910.134)

A reasonable course of action is to solicit bids for the repairs based on different scenarios and compare the cost-benefit of each type of repair. Until you confer with repair contractors, you will not have all the necessary information to make the decision. Furthermore, this advice would hold true for all the suggestions in the report.

The repairs should address the underlying cause(s) of the high moisture problems and not be a short-term solution. All repairs should be performed by a qualified licensed contractor.

Any items found by the repair contractor that are not cited in this report should be repaired as well.

For areas with a high moisture content, the area should be checked with a moisture meter periodically during drying.  The industry recommends that the moisture content of the wood products should be below 17% before rebuilding to prevent fungal growth.

## Health-sensitive occupancy notice

The property is occupied by individuals who reported coughing or throat irritation, nasal congestion or sneezing, sinus problems or headaches, and skin irritation that reportedly improve when away from the home. All repairs and remediation work should therefore be performed in strict compliance with current standards applicable to health-sensitive occupancy.

Applicable Standards and Requirements:

The following standards and guidelines establish the minimum requirements for this project:

- ANSI/IICRC S520:2024 - Standard for Professional Mold Remediation (4th Edition)

- ANSI/IICRC S500:2021 - Standard for Professional Water Damage Restoration (5th Edition)

- ACR 2025 - The NADCA Standard for Assessment, Cleaning and Restoration of HVAC Systems

- EPA Guidelines - Mold Cleanup After Disasters and Indoor Environmental Guidelines

- OSHA Standards - Worker respiratory protection and occupant safety protocols (29 CFR 1910.134)

Enhanced Remediation Requirements:

Work performed under this specification must include:

- Containment Protocols - Complete isolation of work areas using plastic sheeting sealed with duct tape; HVAC systems must be shut down during active remediation to prevent cross-contamination of other areas.

- Air Quality Management - All work areas must be maintained under negative pressure using HEPA-filtered equipment. High-Efficiency Particulate Air (HEPA) filtration is required for dust and spore control.

- Worker Safety and Hygiene - All remediation personnel must be trained and certified in mold remediation (IICRC S520 certification minimum). Personnel must maintain protective protocols including appropriate respiratory protection, full-body coveralls, and equipment handling per OSHA standards.

- Moisture Management - All salvageable building materials must be dried to industry-acceptable moisture levels as specified in ANSI/IICRC S500. Moisture meters must be used to document drying progress and verify completion.

- Material Removal and Disposal - Per ANSI/IICRC S520:2024, source removal is the primary remediation method. Contaminated materials must be physically and completely removed, not merely disinfected or encapsulated. Chemical cleaners, antimicrobial agents, and biocides should only be applied per manufacturer specifications for EPA-registered products and only after physical source removal is complete. Gaseous, vapor-phase, or aerosolized treatments are prohibited due to occupant health concerns and lack of scientific validation per S520:2024 Section 2.

- Post-Remediation Verification - All remediation work must be verified and documented according to S520 standards before occupancy is permitted. Clearance testing or visual inspection protocols must be completed and provided to the building owner.

- HVAC System Restoration - HVAC systems serving affected areas must be restored per IICRC S520:2024 Section 13 using procedures outlined in ACR 2025 The NADCA Standard. All cleaning must include source removal as the primary remediation method, with verification by Surface Comparison Test or NADCA Vacuum Test. If airborne mold spores, mycotoxins, or extracellular matrix (ECM) contamination is confirmed or cannot be ruled out, comprehensive HVAC system restoration to specified cleanliness levels must be performed.

Contractor Notification and Compliance:

Therefore, this property requires work to be performed by contractors with demonstrated expertise in health-sensitive remediation environments. All work must be completed in compliance with these enhanced standards. Failure to meet these requirements may result in property reinfestation, extended remediation costs, and potential occupant health impacts.

All contractors working on this property should acknowledge receipt of this notice and agree to comply with the standards outlined above.

# Limitations

IIS endeavors to perform the required services for the client with the same degree of care and skill ordinarily exercised, under similar circumstances, by members of the profession currently practicing under similar conditions. The degree to which the building and components are assessed depends upon the type of services described in the scope of work and will be performed in a manner specified within the report.

Although reasonable care is exercised in performing these services, this report or investigation cannot be fully or technically exhaustive, nor does it imply that every component can be inspected or that every possible defect or damage can or will be discovered. Although a reasonable attempt has been made to locate suspect fungi (mold) in the areas identified, the inspection techniques used are inherently limited. Our survey was limited to readily accessible materials. With visual assessments, the client agrees to assume all the risk for all conditions that are concealed from view at the time of the assessment or exist in any area excluded from the Assessment by the terms of this agreement.

Please note that the moisture readings included in this report are the raw data recorded by the moisture meters. Moisture levels are affected by ambient weather conditions and other factors; this can result in variations between the readings taken on one day and the readings taken in the same area on another day. The readings provided in this report are accurate indicators of the presence of retained moisture in the materials in the area assessed at that given moment in time. These readings are not represented to be the absolute moisture content of the full thickness of the materials or framing wood.

This report only represents the condition of the structure at the specific locations indicated. Locations were determined by the inspector according to probable areas of possible moisture intrusion and fungal growth in accordance with accepted industry standards. No judgment is intended or given for any areas not found in this report. By conducting a survey and preparing a report, IIS is merely providing an opinion and does not warrant or guarantee the present or future condition of the subject property.

The opinions expressed throughout this report are to a reasonable degree of certainty and probability and are based upon my background, training, and experience in my field of expertise in microbial and indoor environmental inspection, as well as based upon the information set forth in this report and available for my review. This report is intended for the exclusive use of the Client and may not be relied upon by third parties without prior written consent. As a final note, if additional information is provided or work is necessary in this matter, I reserve the right to amend any conclusions and opinions expressed in the report.

This report was prepared on behalf of Investigative Inspection Services, Inc. I hereby certify that I performed the investigation and/or that these documents were prepared by me personally or under my supervision.

Respectfully submitted,

Stanley Yeskolski, President

# Attachments/appendices